# Redux Phone Dryer Operation

📦 For Use with: Phone and Tablet Dryer (📲 Bluetooth App-Connected Dryer) and Phone Dryer (🔘 Push-Button)

How to Use the Redux Dryers:

🔌 Plug in using official Redux/CUI Inc power adapter (for both dryers). Let it warm up for 1–2 minutes.

🧼 Towel off device. Open lid and place device screen-down.

📱 Place phone/tablet/earbud face-down in chamber. Connect charging cable (if available).

📶 Phone and Tablet Dryers - (Bluetooth App-Connected) Open Redux App on store tablet. Tap the machine ID to connect (Bluetooth light turns solid).

📝 Enter basic customer & device info > Tap "Start Drying."

🔒Phone Only Dryers -  (Push-Button Dryer) Close lid and hold start button for 1 second.

⏱️ Treatment completes in 10–45 minutes (up to 45 min for app-connected). Light or app indicates completion.

⚡ If a device doesn't immediately power on, plug into a factory charger for 30 minutes and attempt soft resets.

Troubleshooting (Quick):

- ❌ No power: Use Redux/CUI power supply, check LED, plug directly.

Use only a "Redux" or "CUI Inc / CUI Japan" power supply - Ensure green LED is on the power brick - Plug directly into a wall outlet (avoid surge protectors).

- 📶 Phone and tablet dryer (app-connected) issues: Close app, unplug 2 min, replug, reconnect.

Close app on all tablets - Unplug Redux for 2+ mins - Replug and open app on ONE tablet - If "Unavailable" machine shows, swipe left > Delete > Restart steps

- ❗ Vacuum fail: Remove case, keep cables clear, clean gasket, reseat seal.

Remove phone cases - Ensure device/cables do not touch chamber walls or lid - Clean and fully dry rubber gasket - Reseat gasket evenly - Remove and discard space saver blocks if present.

- 🔘 Phone dryer (push-button) failure to start drying: Reconnect to Demo Wi-Fi, reset via app.

Reconnect to Demo Wi-Fi - Initiate drying from Redux app to reset 150 offline limit.

Support: Redux.com/pages/support | 📧 Support@Redux.com